PSAT User Guide

CycleRAP v2.11  ·  Version 4  (Updated June 2026)

# Path Safety Assessment Tool (PSAT) User Guide

Welcome to PSAT. The tool helps you create projects from geotagged survey images, review coded path segments, calculate safety risk scores, analyse safety hazards and test treatments.

## Table of Contents

## 1. Getting Started

### 1.1 Using the Help Guide

Click the Help (?) button at the top left corner of any page to open the in-app guide. The Help page contains three tabs:

User Guide — step-by-step instructions for all workflows (this guide)

Admin Guide — system deployment, model management, and infrastructure

Developer Guide — technical architecture, API reference, and scoring logic

### 1.2 User Login & Account Management

Creating a new profile — click Create Profile on the Landing Page, enter your LTA Employee Email (e.g. user@lta.gov.sg), your Division, and a 4–12 digit numeric PIN, then click Create Profile.

Signing in — click your profile card on the Landing Page, click Start As \<LTA Email Address\>, enter your PIN, and you are taken to your Projects page.

Switching accounts — click Log Out in the left-hand sidebar at any time and select a different profile (identified by your LTA Employee Email, e.g. user@lta.gov.sg) on the Landing Page.

Session Behaviour

Sessions are device-local. All browser tabs open on the same device share the same active profile.

Logging out from one tab logs you out everywhere on that device.

Your projects, images, and results persist on disk — logging out does not delete any data.

Forgotten PIN

PINs are stored locally and there is no self-service reset. Ask your administrator to delete and recreate the profile if the PIN is irrecoverable. Note that deleting a profile removes all projects associated with it.

### 1.3 Open the Project List

Use the Home page to browse all projects. You can:

search by project name or road name

filter by tags (e.g. NSC, AMK)

sort by project name, verification progress, distance verified, autocode progress, and last modified time

select one or more projects for deletion, coding, path analysis, or treatment work

### 1.4 Create a Project

Use Create Project when you want to build a new project from the source image folders in in/.

You can create a project in three ways:

Single folder — select one source folder directly

Polygon / Planning Area — draw a polygon or click a planning area on the map and create a project from multiple selected roads

Upload area GIS layers — upload area layers to define the project boundary

When creating a project:

enter a project name without underscores

add tags if needed (for easy grouping, e.g. NSC, AMK)

upload images into a source folder if the folder is missing

check that selected roads are marked as available before you create the project

#### Road Highlight on Source Folder Select

When you select a source folder in the Single Folder workflow, PSAT reads the road name from the folder and automatically highlights the matching road on the map in amber/orange. The map also pans and zooms to that road so you can visually confirm you are creating the project from the correct location before proceeding.

### 1.5 Navigate Between Workflows

From the project list, you can send selected projects for the following:

| Workflow | Purpose |
| --- | --- |
| Delete Project | Housekeeping to remove unwanted projects |
| Coding | Detailed attribute review and saving |
| Analyse Projects | Multi-project filtering, charts, generate reports and export data, images, shp files |
| Treatment Application | Before/after scenario testing |

### 1.6 Viewing and Updating GIS Layers

Click View GIS Layers from the sidebar to open the GIS Layers dashboard.

From the GIS Layers page you can:

Browse all current GIS layers (sorted in alphabetical order)

View last updated date, required columns, and metadata for each layer

Filter shapefiles — type any keyword into the filter box beside the "Available Shapefiles" heading to instantly narrow the list to matching layers. This is useful when you have many layers and want to jump straight to the one you need (e.g. type bus to surface all bus-related layers)

Rename a layer — click Edit next to any shapefile to give it a custom display name that is meaningful to your team. The rename takes effect immediately across the dashboard

Revert to original name — if a layer has been renamed, click Revert to restore its original filename at any time

Using the Update GIS Layer button:

Add entirely new GIS layers for new categories

Replace an existing layer with an updated file

Delete a layer that is no longer needed

For full GIS layer management instructions, see [Section 8: GIS Layer Management](#8-gis-layer-management).

## 2. Coding Page

The Coding page is the main review workspace. It can open one or more selected projects in a combined session.

## Table of Contents

### 2.1 Main Layout

The page keeps three views in sync:

the current segment image

the attributes table

the segment map

Selecting a segment in one area automatically updates the others.

### 2.2 Navigating Segments

You can navigate through segments in three ways:

Type a segment number in the jump box to go directly to that segment

Click Next or Back to move one segment at a time

Click a point on the segment map to select it

### 2.3 Attribute Default Values

When a segment is first created and no auto-code has been run, PSAT assigns the following default values. These represent the most common parameters for paths in Singapore.

| # | Attribute | Default Value | Reason |
| --- | --- | --- | --- |
| 1 | Area Type | Suburban | GIS fallback using URA land use layers; default when no urban/industrial/recreational polygon match |
| 2 | Facility Type | Sidewalk | Most common facility type; overridden by CV image analysis |
| 3 | Facility Access | Adequate | Most common; code Inadequate only when congestion or blockages force users onto alternative routes or onto the roads |
| 4 | Loose or Slippery Surface | Not Present | Most common; GIS-assisted via PATH defects layer (algae); CV detection not yet ready |
| 5 | Tram or Train Rails | Not Present | No tram or train rail paths/roads exist in Singapore |
| 6 | Major Surface Deformation or Drain Opening | Not Present | Most common; GIS-assisted via PATH defects layer (tile cracks, tactile tile cracks, uneven pavement) |
| 7 | Fixed Obstacle on Facility | Not Present | Most common; overridden by CV image analysis |
| 8 | Non-Fixed Obstacle on Facility | Not Present | Most common; overridden by CV image analysis |
| 9 | Line of Sight | Adequate | Not yet in auto scoring specification; requires manual assessment of visibility along the path |
| 10 | Delineation | Not Present | Default not present as the default Facility Type = Sidewalk; CV codes first, then overwritten if PATH provides faded marking data |
| 11 | Light Segregation | Present | All off-road paths are expected to have kerb segregation; confirmed by CV and logic rules |
| 12 | Facility Width per Direction | Narrow | GIS-derived from path width data based on Dec 2024 LiDAR scan; default when no width data available |
| 13 | Flow Direction | Two Way | Most on-road cycling lanes are one-way; all off-road paths default to two-way |
| 14 | Width Restriction | Not Present | Most common; logic rule sets Present when a fixed or non-fixed obstacle is detected and occupy >=20% of the path clear width |
| 15 | Adjacent Road Lane 0–1m | Not Present | CV-derived; present when path edge is ≤1m from road kerb with no protective barrier |
| 16 | Adjacent Vehicle Parking 0–1m | Not Present | Most common; GIS-derived from URA and HDB car park lot layers |
| 17 | Adjacent Severe Hazard 0–1m | Not Present | Rare in Singapore; requires manual assessment when waterways or drops >60cm are present with no barrier |
| 18 | Adjacent Object/Level Change 0–1m | Not Present | Off road paths should have adjacent object/level change present but depends if the road carriageway is 0-1m from the path edge (without barrier). inferred by CV and logic rules |
| 19 | Adjacent Sidewalk 0–1m | Not Present | derived by CV and logic rules |
| 20 | Adjacent Road Lane 1–3m | Not Present | CV-derived; present when road is 1–3m from path with no protective barrier |
| 21 | Adjacent Vehicle Parking 1–3m | Not Present | Most common; GIS-derived from URA car park lot layer |
| 22 | Adjacent Severe Hazard 1–3m | Not Present | Rare in Singapore; requires manual assessment when severe hazards are 1–3m from path |
| 23 | Adjacent Object/Level Change 1–3m | Not Present | Co-occurs with adjacent road lane; inferred by CV and logic rules |
| 24 | Adjacent Sidewalk 1–3m | Not Present | Not present by default; CV and logic rules detect when split paths with >1m green verge are present |
| 25 | Grade | < 5 Degrees | Most paths in Singapore are flat; GIS-derived from LiDAR 3D point cloud data |
| 26 | Curvature | No Sharp Turn Present | Most common; GIS-derived using path geometry and curvature radius calculation |
| 27 | Street Lighting | Present | Most urban paths in Singapore have adequate street lighting |
| 28 | Pedestrian Crossing | Not Present | Most common; GIS-derived from MRT exit and bus stop layers; manual coding where needed |
| 29 | Intersecting Bicycle Facility | Not Present | Most common; GIS-derived using pedestrian crossing layer within 5m radius |
| 30 | Intersection Approach | Separate/NA | Most off-road cycling paths remain separate from traffic at intersections |
| 31 | Intersection or Road Crossing | Not Present | Most common; overridden by CV when crossings are detected |
| 32 | Crossing Facility | Not Present | Most common; overridden by CV when zebra crossings, signalized crossings or bicycle crossings are detected |
| 33 | Number of Lanes – Adjacent Road | > 1 per Direction | Most common; GIS-derived from LiDAR kerbline layer |
| 34 | Number of Lanes – Intersecting Road | 1 per Direction/NA | Default NA when no intersection is present; GIS-derived from LiDAR kerbline layer when intersection exists |
| 35 | Property Access | Not Present | Most common; CV and GIS auto-coding pending |
| 36 | Peak Pedestrian Flow | None | GIS-derived from MRT exit, bus stop, and AMG sensor count layers; default when no count data available |
| 37 | Peak Bicycle/LV Traffic Flow | Low | GIS-derived from AMG 24/7 sensor count data; default when no count data available |
| 38 | Observed Proportion of Cargo Bikes | Low | Most common; ePMD and PAB in Singapore are under 20kg, classifying them as low cargo proportion |
| 39 | Bicycle/LV Speed – Average | < 20 km/h | Most common cycling speed on Singapore shared paths and CPNs; Pending future AM device speed data |
| 40 | Bicycle/LV Speed Differential | < 10 km/h | Most common; low speed variation typical on Singapore shared paths |
| 41 | Road AADT | 6000 | Must be coded manually; no GIS auto-code path; ERP2 data integration pending |
| 42 | Heavy Vehicle Flow | Low | Most paths are away from heavy traffic; GIS-derived from bus lane marking layer (Geospace) |
| 43 | Road Speed Limit | 10 km/h | GIS-derived from Geospace speed limit layer; used for analysis only and does not affect risk scores |
| 44 | Road Operating Speed (mean) | 30 | GIS-derived from LinkID layer and ERP2 speed data; required when adjacent road or road crossing is present |

### 2.4 Auto-Code Options

PSAT supports five auto-code methods that can be run individually or in combination:

CV auto-code — reads the segment image using computer vision models

GIS auto-code — reads GIS shapefiles for the segment location

Logic rules — applies cascade rules based on detected surface types

Auto-code (by segment) – auto-code all attributes by the selected segment

Bulk auto-code — auto-code all attributes for all segmentsacross selected project(s)

Per-attribute auto-code — targets only certain attributes across selected project(s)

Autocode progress is tracked in the project listing as Percentage Segments Autocoded.

#### 2.4.1 Attributes Coded by CV (Image Analysis)

The following attributes are automatically inferred from street-level photographs using YOLO computer vision models:

| Attribute | How CV Determines the Value |
| --- | --- |
| Facility Type | Off-road bicycle classifier + fixed obstacle decision logic |
| Light Segregation | Set to Present by default when any path is detected |
| Adjacent Road Lane 0–1m | Adjacent road classifier (confidence ≥ 0.8) |
| Adjacent Road Lane 1–3m | Adjacent road classifier (confidence ≥ 0.8) |
| Adjacent Object/Level Change 0–1m | Inferred from Adjacent Road Lane 0–1m result |
| Adjacent Object/Level Change 1–3m | Inferred from Adjacent Road Lane 1–3m result |
| Adjacent Sidewalk 0–1m | Multi-path detection (multiple path segments visible) |
| Fixed Obstacle on Facility | Fixed obstacle segmentation model |
| Non-Fixed Obstacle on Facility | Fixed obstacle segmentation (label 9 = non-fixed) |
| Delineation | Delineation classifier model |

#### 2.4.2 Attributes Coded by GIS Layer Mapping

The following attributes are automatically derived from the GIS shapefiles stored in the system:

| Attribute | GIS Layer Used | Buffer / Method |
| --- | --- | --- |
| Area Type (Urban/CBD) | area_type polygon | Point-in-polygon containment |
| Area Type (Industrial) | area_type polygon | Point-in-polygon containment |
| Area Type (Rural) | LanduseRural2026 / rural polygon | Point-in-polygon containment |
| Area Type (Recreational) | LanduseRecre2026 / recreation polygon | Point-in-polygon containment |
| Area Type (Suburban) | *(No layer match)* | Default fallback |
| Facility Width per Direction | path / CyclingPath_Jul2024 / FootPath_Mar2025 | Nearest within 5 m |
| Road Operating Speed (mean) | LinkID_Shape_File (with speed CSV lookup) | Nearest road link |
| Road Speed Limit | Roadattribute Speed_limit | Nearest road link |
| Number of Lanes – Adjacent Road | kerb_line | Nearest within 10 m |
| Adjacent Vehicle Parking | HDB Car Park Lots/ URA Parking Lot | Within 20 m buffer |
| Peak Pedestrian Flow | AMGbeforeCount/ AMGsensorCount/ Bus Stop/ Bus Shelter/ MRT exit | Within 20 m buffer |
| Peak Bicycle/LV Traffic Flow | AMGbeforeCount / AMGsensorCount | Within 20 m buffer |
| Crossing Facility | roadcrossinglayer / /AMG_BC2025_shp | Within 2 m buffer |
| Pedestrian Crossing | Mrt_exit / bus_stop / bus shelter / Road crossing | Within 20 m buffer |
| Heavy Vehicle Flow | Lane marking Type A1, A6(bus_lane) proximity | Within 20 m buffer |

**Note:** Road AADT has no GIS auto-coding path — it must be coded manually.

#### 2.4.2.1 Grade

- Data cleaning

Verify LiDAR points for gaps or corrupted files.

Remove any points without elevation value

Remove points below -10m and +65m (Bukit Timah Hill still lies below this point so there's no reason for 170m)

LiDAR elevation range filter

Obtain all LiDAR points within 3m of identified GPS point

Assign these LiDAR points a fixed score multiplier based on distance from point

Score in relation to distance is calculated using this sigmund curve modified to accept a parameter for number of points

Confidence level is not a strict cut-off point but instead a point at which the score begins to diminish rapidly, seen by the first curve of the sigmoid below. A higher confidence inversely affects the severity of the curve, resulting in a flatter score distribution and thus more evenly weighted across all distance, i.e. a high confidence of 100 will effectively cause points at 0m and 3m to have identical weightage. The confidence level of 2.87 is determined to be the highest accuracy based on comparisons to testing with personal equipment in a selected area (e.g. smartwatch's baro altimeter).

Sigmoid curve for point score calculation

Plot all points on a 3d graph such that Y = elevation, X = distance and Z = score

Within Singapore, the expected range is 0m to 15m.

Determine elevation using simple weighted aggregation

Points closer to 0m have higher weightage in regards to elevation, based on computed score given the sigmoid function above. To be more specific, inverse distance weighting formula below is being used where E-hat is the resulting estimated elevation (the hat signifies estimated, wi being the weight of point i, and ei being the elevation of point i. Further clarification: weight = score, the number calculated from the sigmoid curve above.

Inverse distance weighting formula

REASONING

3m search radius balances spatial specificity against the risk of insufficient point density in any single capture

inverse distance weighting is an established spatial interpolation technique that preserves the intuition that nearer measurements are more representative of a target location

logistic function is used to model confidence as a function of point count because density exhibits diminishing returns on estimate reliability i.e the difference between 1 and 10 points is significant, while the difference between 100 and 110 is negligible

Weighted aggregation over distance-scaled scores produces a single elevation estimate that is robust to outlier points at the periphery of the search radius

#### 2.4.2.2 Curvature

Path is snapped to nearest GIS path centreline; uses the facility under this priority cycling>shared>footpath, i.e. if side by side cycling path/footpath, curvature will always use cycling path

Buffer of 5m radius to prevent across the road problems

5m buffer zone diagram

ALONG PATH

Within 5m (this is the radius we see) the path line found previously is densified to 0.5m spaced points and slides in consecutive triplet points (as in 0m is point A, 0.5m is point B, etc), one area can have many of these triplets

The algorithm fits a circle that passes through any 3 points using this: R = (a * b * c) / (4 / Area) where area is derived from Heron's formula

Passes through these rejection criteria: Nearly straight (collinear) abc points skipped (straight), micro-triangle abc points under 0.35m each side is skipped (inaccurate), lopsided abc (one long side one short side etc) skipped (inaccurate). Exact numbers are if long end is 3x the short end

Points ABC form lines AB and BC. Lopsided 3x signifies that AB:BC > 1:3 or vice versa.

!Triplet lopsided rejection diagram

Now we have a few non-rejected abc triplets, use the worst case (minimum radius)

PATH INTERSECTION

If no sharp turn, run this, so if got sharp turn + path intersection, always will show sharp turn

Any vertex deflection angle > 45 deg

Must have >1m arm lengths (to account for drawing error)

!Path intersection deflection angle diagram

EXACT NUMBER JUSTIFICATIONS

0.5m densify spacing: any smaller, detection becomes too sensitive. Any higher, not enough points. Each triplet here spans roughly 1m and 5-6deg arc of a 10m circle

0.35m min triplet leg: GIS data shows this to be the breaking point where any smaller, centreline noise/inaccuracy starts to take ove

#### 2.4.3 Attributes Coded by Logic Rules

Logic rules apply a cascade system based on what surface types the CV model detects in the image. Each step overrides the previous if its trigger condition is met:

| Step | Trigger Condition | Key Attributes Set |
| --- | --- | --- |
| Step 1 — Default | Always applied first | Facility Type = Sidewalk, Light Seg. = Present, Delineation = Not Present |
| Step 2 — Cycling Path | Cycling/Wet Cycling surface in bottom 20% of image | Facility Type = Off-Road Bicycle Path, Delineation = Present |
| Step 3 — Red Stripe | Red Stripe surface in bottom 20% of image | Facility Type = Multi-Use Path, Delineation = Present |
| Step 4 — Traffic Crossing | Traffic Crossing markings ≥ 80% of bottom 10% | Facility Type = Mixed Traffic, Crossing Facility = Present, Intersection = Present |
| Step 5 — Zebra Crossing | Zebra crossing ≥ 80% of bottom 10% | Same as Step 4 |
| Step 6 — Road Surface | Road pixels ≥ 80% of bottom 10% | Facility Type = Mixed Traffic, Light Seg. = Not Present, Adj. Road 0–1m = Present |

### 2.5 Manual Review

You can override any coded value directly in the table. The page also shows:

risk score updates (on-the-fly) for the selected segment as you change values

field-source provenance showing whether each value came from CV, GIS, logic rules, or manual entry

boxed attributes highlighting fields that have been manually overwritten

a validation summary table comparing the percentage of attributes overwritten against the stored autocoded baseline

field-source provenance showing whether each value came from CV, GIS, logic rules, or manual entry

### 2.6 Map Preview & Analysis

For supported attributes, the page can show extra spatial detail within a 5m radius of the current segment:

nearby GIS layers (e.g. cycling path, footpath, MRT stations, bus stops, land ownership)

curvature visualization

width visualization

grade or gradient details when profile data is available

#### Auto-enable GIS Layers on Analysis Overlay

When you turn on the Analysis Overlay toggle on the coding page map, PSAT automatically enables the Footpath, Cycling Path, and Shared Path GIS layers so the overlay is always shown over visible path geometry. There should be a black hollow circle (with 5m radius) showing the features (i.e. radius, width) snapped to the image.

#### Filtered Segments from Path Analysis

If you navigate to the Coding page directly from Path Analysis (by clicking a segment on the Path Analysis map), the coding page map will show only the segments that were visible in your active filter. The currently selected segment is always shown regardless of the filter. This makes it easier to focus on a specific subset while coding without losing your analysis context.

### 2.7 Save and Progress Tracking

After review:

save your attribute edits to persist them

update the Segments Verified Percentage counter (on top) as you complete manual checks

### 2.8 CycleRAP Reference

The CycleRAP button (next to Coding Guide in the top tab bar) opens the official iRAP CycleRAP page at irap.org/cyclerap in a new browser tab. Use it to look up for attribute definitions and rationale, scoring rationale, or the full CycleRAP methodology.

#### What is CycleRAP?

CycleRAP is the international standard for evaluating road and cycling infrastructure safety. It assesses risk for bicyclists and other light mobility users across all facility types — on-road or off-road — without requiring crash data. PSAT's scoring engine is built on the CycleRAP methodology.

#### Resources available on the CycleRAP page

Once the page opens, you will find four downloadable or interactive resources:

| Resource | What it contains |
| --- | --- |
| Download CycleRAP Methodology | The full technical specification — all attributes, scoring multipliers, crash type formulas, and risk band thresholds used in PSAT |
| Download CycleRAP User Guide | A practical step-by-step guide for surveyors and coders on how to apply CycleRAP in the field |
| Explore the CycleRAP Demonstrator Tool | An interactive online tool showing sample assessments and how scores are calculated |
| Where is CycleRAP being used? | A map and list of countries and organisations currently using CycleRAP |

**Tip:** The attribute names in PSAT map directly to the CycleRAP Methodology. If you are unsure what value to assign to an attribute (e.g. *Facility Width per Direction* or *Peak Pedestrian Flow*), the Methodology PDF contains the exact definitions and example photographs for each option.

### 2.9 Hover Tips — Safety Scores & Attributes

PSAT surfaces contextual help through hover tooltips throughout the interface.

#### Crash Type Score Tooltips

On any page that shows the Crash Type Scores panel (Coding Page, Path Analysis, Treatment Page), hover over any of the five score cards to see the risk banding thresholds for that crash type.

BB / BP / SB (Bicycle-Bicycle, Bicycle-Pedestrian, Single-Bicycle)

| Band | Score Range |
| --- | --- |
| Low | < 5 |
| Medium | 5 – 10 |
| High | 10 – 20 |
| Extreme | > 20 |

VB (Vehicle-Bicycle)

| Band | Score Range |
| --- | --- |
| Low | < 10 |
| Medium | 10 – 25 |
| High | 25 – 60 |
| Extreme | > 60 |

The Risk Score is the sum of all four crash type scores. Its banding colour reflects the worst-case band across all crash types. Hovering the Risk Score card shows a compact summary of all banding thresholds for reference.

**Tip:** Tooltips appear instantly on hover and stay visible even if you accidentally click the card. Move the cursor away to dismiss.

#### Attribute Info Tooltips

On the Coding Page, every coding attribute that has a description shows a small ⓘ info icon next to its label. Hover the icon to read a plain-English explanation of:

What the attribute measures.

How it is typically coded in a Singapore context.

How it contributes to the risk score.

Example:

**Area type** — *"Classify the surrounding land use. Singapore paths are mostly Suburban (HDB/residential). Use Urban for city area and dense commercial zones, Industrial for business parks and logistics areas, Recreational for parks."*

#### Where Tooltips Appear

| Location | What is shown |
| --- | --- |
| Crash Type Score cards (BB/BP/SB/VB) | Risk banding thresholds for that crash type |
| Risk Score card | Full banding summary for all crash types |
| Attribute ⓘ icons (Coding Page) | Plain-English description of the attribute and scoring impact |

### 2.10 Attribute Logic Checks on Verification

When you mark a segment as Verified, PSAT checks the coded attributes for contradictory or impossible combinations. There are two categories of checks.

#### Hard Rules

Hard rules flag combinations that cannot coexist. Resolve all hard-rule conflicts before marking a segment as Verified.

| # | If | Then |
| --- | --- | --- |
| 1 | Adjacent Road Lane 1–3m = Present | Adjacent Road Lane 0–1m = Not Present |
| 2 | Adjacent Vehicle Parking 1–3m = Present | Adjacent Vehicle Parking 0–1m = Not Present |
| 3 | Adjacent Severe Hazard 1–3m = Present | Adjacent Severe Hazard 0–1m = Not Present |
| 4 | Adjacent Object or Level Change 1–3m = Present | Adjacent Object or Level Change 0–1m = Not Present |
| 5 | Adjacent Sidewalk 1–3m = Present | Adjacent Sidewalk 0–1m = Not Present |
| 6 | Facility Type = Sidewalk | Adjacent Sidewalk 0–1m = Not Present |
| 7 | Facility Type = Mixed Traffic Road Lane | Adjacent Road Lane 0–1m = Present and Adjacent Road Lane 1–3m = Not Present |
| 8 | Facility Type = Sidewalk, Multi-Use Path, or Off-Road Bicycle Path | Adjacent Object or Level Change = Present |
| 9 | Width Restriction = Present | Fixed Obstacle on Facility = Present or Non-Fixed Obstacle on Facility = Present (or both) |
| 10 | Facility Type = Sidewalk, Multi-Use Path, or Off-Road Bicycle Path | Light Segregation = Present |
| 11 | Property Access = Present | Intersection or Road Crossing = Present |
| 12 | Facility Type = On-Road Bicycle Lane | Adjacent Road Lane 0–1m = Present |
| 13 | Adjacent Road Lane 0–1m = Present | Road AADT ≠ 0 |
| 14 | Adjacent Road Lane 0–1m = Present | Road Operating Speed (mean) ≠ 0 |
| 15 | Adjacent Road Lane 1–3m = Present | Road AADT ≠ 0 |
| 16 | Adjacent Road Lane 1–3m = Present | Road Operating Speed (mean) ≠ 0 |
| 17 | Intersection or Road Crossing = Present | Road AADT ≠ 0 |
| 18 | Intersection or Road Crossing = Present | Road Operating Speed (mean) ≠ 0 |
| 19 | Facility Type = Mixed Traffic Road Lane | Road AADT ≠ 0 |
| 20 | Facility Type = Mixed Traffic Road Lane | Road Operating Speed (mean) ≠ 0 |
| 21 | Facility Type = Road Shoulder | Adjacent Road Lane 0–1m = Present |

**Rules 1–5** reflect the spatial logic that if a hazard exists in the outer band (1–3m) it cannot simultaneously exist in the inner band (0–1m) for the same feature type.

**Rules 13–20** reflect that Road AADT and Road Operating Speed must have non-zero values whenever an adjacent road lane or road crossing is present, or when the facility itself is on a road.

## 3. Map View & Analysis

Map views appear in several PSAT workflows, especially Coding, Path Analysis, Treatment, and GIS Layers.

## Table of Contents

### 3.1 Segment Map in Coding

On the Coding page, the map helps you:

click a segment to select it

see risk-band colouring for saved scoring results

view nearby GIS context for the current point

inspect the spatial position of the current segment

### 3.2 Risk Colour Bands

Segments are colour-coded by their highest relevant risk band so you can quickly spot high-risk areas.

PSAT calculates four independent risk scores per segment:

| Score | Crash Type |
| --- | --- |
| BB | Bicyclist–Bicyclist |
| BP | Bicyclist–Pedestrian |
| SB | Single Bicyclist (departure/fall) |
| VB | Vehicle–Bicyclist |

#### Risk Bands for BB, BP, SB

| Band | Label | Score Range | Map Colour | Hex |
| --- | --- | --- | --- | --- |
| 1 | Low | < 5 | 🟢 Green | #87C424 |
| 2 | Medium | 5 – 10 | 🟡 Yellow | #FFCC1A |
| 3 | High | 10 – 20 | 🟠 Orange | #FF5B1A |
| 4 | Extreme | > 20 | 🟣 Purple | #CD1AFF |

#### Risk Bands for VB (Vehicle–Bicyclist)

| Band | Label | Score Range | Map Colour | Hex |
| --- | --- | --- | --- | --- |
| 1 | Low | < 10 | 🟢 Green | #87C424 |
| 2 | Medium | 10 – 25 | 🟡 Yellow | #FFCC1A |
| 3 | High | 25 – 60 | 🟠 Orange | #FF5B1A |
| 4 | Extreme | > 60 | 🟣 Purple | #CD1AFF |

**Overall Risk Level Band** = the **highest** band across BB, BP, SB, and VB for that segment. A single Extreme sub-score makes the overall band Extreme regardless of the others.

### 3.3 GIS Context

Depending on the page and workflow, PSAT can show nearby GIS information within a 5 m radius of the current segment, such as:

cycling paths, shared paths, and footpaths

road crossings and zebra crossings

bus stops and MRT-related features

road-name and planning-area context

width, curvature, and gradient analysis overlays

path defects

land ownership

### 3.4 GIS Layers Page

Use the dedicated GIS Layers page (accessible from the sidebar) when you want to inspect the raw shapefile layers themselves rather than just the map context around a segment.

That page lets you preview available layers, review their metadata, and add or replace GIS files when needed.

## 4. Path Analysis

The Path Analysis page is the multi-project analysis workspace.

## Table of Contents

### 4.1 Select Projects

You can select one or more projects to analyse together. The search box matches both project names and source road names, so you can find projects even when the project title and road name differ.

### 4.2 Filter Before Loading

Before loading projects, you can filter by:

| Filter | Description |
| --- | --- |
| Project or road name | Text search |
| Tag | e.g. NSC, AMK, Pre, Post |
| Date created range | Filter projects created within a date window |
| Last updated range | Filter projects updated within a date window |

### 4.3 Analyse Loaded Data

After loading projects, you can:

choose up to five attributes to focus on

apply segment-level filters (see sections below)

view attribute distributions and aggregated score-band summaries

inspect filtered segments on the map and in the table

click a segment to navigate to the Coding page to make edits

click Back to Analysis to return and continue

#### 4.3.1 Attribute Filters and Finer Filtering

When you select an attribute for analysis, you can filter segments by its coded values. Attributes marked ❖ also support finer filtering — selecting a specific value (e.g. *Present* for Fixed Obstacle on Facility) reveals a secondary sub-category dropdown so you can pinpoint exact sub-types. The map updates to show distinct colours for each sub-category once a sub-type is selected.

| Attribute | Filter Values | Finer Filter Sub-categories (❖ = available) |
| --- | --- | --- |
| Facility Type | Sidewalk; Multi-Use Path; Off-Road Bicycle Path; On-road Bicycle Lane; Road Shoulder; Mixed Traffic Road Lane | — |
| Area Type | Urban/CBD; Suburban; Rural; Industrial; Recreational | — |
| Adjacent Road Lane 0–1m | Present; Not Present | — |
| Adjacent Road Lane 1–3m | Present; Not Present | — |
| Adjacent Vehicle Parking 0–1m | Present; Not Present | — |
| Adjacent Vehicle Parking 1–3m | Present; Not Present | — |
| Facility Width per Direction ❖ | Very Narrow; Narrow; Wide | Very Narrow: ≤1.5m; >1.5–1.8m; >1.8–<2m · Narrow: 2–<3.5m; 3.5–4m · Wide: >4m |
| Flow Direction | One Way; Two Way | — |
| Grade | ≤2° (1:25); 2.9° (1:20); 3.8° (1:15); 4.7° (1:12); ≥5° | — |
| Curvature ❖ | Sharp Turn Present; No Sharp Turn Present | Sharp Turn: <6.5m; 6.5–<10m; Path Junction · No Sharp Turn: 10–18m; >18m |
| Street Lighting | Present; Not Present | — |
| Delineation ❖ | Present; Not Present | When "Present": Cycling Path; Red Stripe; Signalised Crossing; Traffic Crossing; Zebra Crossing <br> When "Not Present": Faded Marking |
| Fixed Obstacle on Facility ❖ | Present; Not Present | When "Present": Lamp Post; Traffic Light; Pillar; Bollards; Fence; Vegetation; Others |
| Non-Fixed Obstacle on Facility ❖ | Present; Not Present | When "Present": Barrier; Bins; Bicycle; Cone; Others |
| Light Segregation | Present; Not Present | — |
| Intersection or Road Crossing | Present; Not Present | — |
| Crossing Facility ❖ | Present; Not Present | When "Present": Zebra Crossing; Signalised PC; Bicycle Crossing; Unsignalised Junction; Development Access |
| Property Access | Present; Not Present | — |
| Tram or Train Rails | Present; Not Present | — |
| Major Surface Deformation or Drain Opening ❖ | Present; Not Present | When "Present": Footpath crack; Footpath uneven; Footpath Uneven IMU |
| Peak Pedestrian Flow | None; Low; Moderate to High | — |
| Peak Bicycle/LV Traffic Flow | Low; Moderate to High | — |
| Observed Proportion of Cargo Bikes | Low; Moderate to High | — |
| Heavy Vehicle Flow | Low; Moderate to High | — |
| Bicycle/LV Speed – Average | < 20 km/h; ≥ 20 km/h | — |
| Road Speed Limit | NA; 30 km/h; 40 km/h; 50 km/h; 60 km/h; 70 km/h; 80 km/h; 90 km/h | — |
| Overall Risk Level Band | 1 (Low); 2 (Medium); 3 (High); 4 (Extreme) | — |

#### 4.3.2 Finer Filtering — Sub-category Options

Selecting a top-level filter value for any attribute marked ❖ reveals a secondary sub-category dropdown. Each sub-category is shown with a distinct colour on the map.

| Attribute | Trigger Value | Sub-category Options |
| --- | --- | --- |
| Facility Width per Direction | Very Narrow | ≤1.5 m; >1.5–1.8 m; >1.8–<2 m |
| Facility Width per Direction | Narrow | 2–<3.5 m; 3.5–4 m |
| Facility Width per Direction | Wide | >4 m |
| Curvature | Sharp Turn Present | <6.5 m (footpath threshold); 6.5–<10 m; Path Junction |
| Curvature | No Sharp Turn Present | 10–18 m; >18 m (cycling path threshold ≥18 m) |
| Fixed Obstacle on Facility | Present | Lamp Post; Traffic Light; Pillar; Bollards; Fence; Vegetation; Others |
| Non-Fixed Obstacle on Facility | Present | Barrier; Bins; Bicycle; Cone; Others |
| Delineation | Present | Cycling Path; Red Stripe; Signalised Crossing; Traffic Crossing; Zebra Crossing |
| Delineation | Not Present | Faded Marking |
| Crossing Facility | Present | Zebra Crossing; Signalised PC; Bicycle Crossing; Unsignalised Junction; Development Access |
| Major Surface Deformation or Drain Opening | Present | Footpath crack; Footpath uneven; Footpath Uneven IMU |

How to use finer filtering:

In the Filter panel, select an attribute that supports finer filtering (marked ❖).

Choose a top-level value (e.g. *Present* for Fixed Obstacle on Facility).

A second dropdown appears — select the specific sub-type you want to highlight.

The map immediately updates to colour only segments matching that sub-type.

To adjust or clear the sub-filter, click the × on the active sub-category chip, or change the top-level value.

**If the finer filtering colours are not appearing correctly on the map**, it is likely because the source attribute was coded incorrectly for some segments. To fix this:

1. Click the affected segment on the map to open it in the Coding page.

2. Review and correct the attribute value (e.g. change the sub-type for Fixed Obstacle Type).

3. Save the segment, then click **Back to Analysis** to return.

4. Re-apply the filter — the updated colour will now be reflected on the map.

### 4.4 Export

The page supports three exports:

| Export | Format | Contents |
| --- | --- | --- |
| Download Table | CSV | All currently filtered segment rows |
| Download Images | ZIP | All images for the currently filtered segments |
| Download Shapefile | ZIP (Shapefile) | Filtered segments exported as point geometry with attributes |

Download Shapefile exports the currently visible filtered segments as a standard GIS shapefile package (.zip). The file is named shapefile_export_YYYY-MM-DD.zip and can be opened in QGIS, ArcGIS, or any other GIS application. Only segments that have a valid image reference are included.

### 4.5 Session Continuity

Your selections and filters are kept for the browser session, so navigating away (e.g. to edit a segment in Coding) and returning does not immediately clear the analysis setup.

### 4.6 Finer Filtering Reference

Finer filtering lets you narrow a segment filter down to a specific sub-type within a parent category. When an attribute supports it, selecting a top-level value reveals a second dropdown — and the map immediately updates to colour only the segments that match your exact sub-type.

Attributes that support finer filtering are marked ❖ in the filter panel.

#### Which Attributes Support Finer Filtering

| Attribute | Top-level Values | Sub-category Count |
| --- | --- | --- |
| Facility Width per Direction ❖ | Very Narrow; Narrow; Wide | 6 width ranges |
| Curvature ❖ | Sharp Turn Present; No Sharp Turn Present | 5 radius ranges |
| Fixed Obstacle on Facility ❖ | Present; Not Present | 7 obstacle types |
| Non-Fixed Obstacle on Facility ❖ | Present; Not Present | 5 obstacle types |
| Delineation ❖ | Present; Not Present | 5 delineation types when Present; 1 type when Not Present |
| Crossing Facility ❖ | Present; Not Present | 5 crossing types |
| Major Surface Deformation or Drain Opening ❖ | Present; Not Present | 3 deformation types |

All other attributes use standard single-level filtering.

#### How to Use Finer Filtering

Open Path Analysis and load one or more projects.

In the Filter Segment panel, select an attribute marked ❖ from the dropdown.

Choose a top-level value (e.g. *Present* for Crossing Facility).

A sub-category dropdown appears immediately below — select the specific sub-type you want (e.g. *Zebra Crossing*).

The map updates to colour only segments with that sub-type. Segments with other values are shown in grey.

To change the sub-category, open the second dropdown and choose a different option.

To clear finer filtering, click the × on the active filter chip, or reset the top-level value to *Not Selected*.

#### Sub-category Reference Table

| Attribute | Trigger Value | Sub-category Options | Map Colour Logic |
| --- | --- | --- | --- |
| Facility Width per Direction | Very Narrow | ≤1.5 m · >1.5–1.8 m · >1.8–<2 m | Red → Orange → Amber (narrower = more risk) |
| Facility Width per Direction | Narrow | 2–<3.5 m · 3.5–4 m | Green shades |
| Facility Width per Direction | Wide | >4 m | Blue |
| Curvature | Sharp Turn Present | <6.5 m (footpath threshold) · 6.5–<10 m · Path Junction | Red · Orange · Purple |
| Curvature | No Sharp Turn Present | 10–18 m · >18 m (cycling path threshold ≥18 m) | Green · Blue |
| Fixed Obstacle on Facility | Present | Lamp Post · Traffic Light · Pillar · Bollards · Fence · Vegetation · Others | Unique colour per type |
| Non-Fixed Obstacle on Facility | Present | Barrier · Bins · Bicycle · Cone · Others | Unique colour per type |
| Delineation | Present | Cycling Path · Red Stripe · Signalised Crossing · Traffic Crossing · Zebra Crossing | Unique colour per type |
| Delineation | Not Present | Faded Marking | Unique colour per type |
| Crossing Facility | Present | Zebra Crossing · Signalised PC · Bicycle Crossing · Unsignalised Junction · Development Access | Unique colour per type |
| Major Surface Deformation or Drain Opening | Present | Footpath crack · Footpath uneven · Footpath Uneven IMU | Unique colour per type |

#### Correcting Wrong Sub-category Colours

If the map shows unexpected colours when a sub-category filter is active, this usually means the source attribute was coded with an incorrect or missing sub-type value.

Click the affected segment on the map to select it.

Click Open in Coding (or navigate to the Coding page from the sidebar).

In the attribute table, find the relevant attribute (e.g. *Fixed Obstacle Type* for Fixed Obstacle on Facility).

Correct the value to the appropriate sub-type from the dropdown.

Click Save to persist the change and recalculate risk scores.

Click Back to Analysis in the sidebar to return to Path Analysis.

Re-apply the same filter — the corrected colour will now appear on the map.

This also applies when the CV auto-coder assigns a sub-type incorrectly. The Coding page lets you override any auto-coded value and is the authoritative source for what appears in the filters.

## 5. Treatment Application

The Treatment Application page lets you test safety improvements on one or more cycling path projects. You can explore which interventions apply to each segment, preview how they would change risk scores, apply them, and compare before-and-after risk distributions — all without permanently altering the underlying survey data until you are ready.

## Table of Contents

### Step 1: Find and load your projects

Project selection is done from the Projects page. Select one or more projects and click the green Treatment Application button to load them into the treatment workspace.

Click a row (or tick its checkbox) to select it. Use the Select All row to select all visible projects at once.

Use the Search box or Filter by tag dropdown to find projects more quickly.

You can load multiple projects together to compare or apply treatments across them in one session.

The Verification Status column shows ✅ for fully verified projects and ⏳ for projects still in progress.

Click the pencil icon in the Actions column to rename a project or change its tags before loading.

### Step 2: Understanding the workspace layout

After loading, the workspace is divided into several areas:

Project tabs (shown at the top when you load more than one project) — By default, the workspace starts on the All Projects tab to show all loaded projects together. Clicking a specific project tab will focus on that project, jumping to its first segment. On the maps, segments from all other projects will automatically grey out, highlighting only the active project.

Before Treatment and After Treatment maps — a side-by-side pair of maps at the top of the page, showing all loaded segments colour-coded by risk level. The right map updates in real time as you apply treatments.

A three-column panel below the maps — Treatment Options on the left, the segment street-level image in the middle, and the Scores & Attributes panel on the right.

An Overall Treatment Analysis section at the bottom with pie charts comparing the risk band distributions before and after your treatments. The charts and percentages dynamically update based on your active tab — showing aggregate data for "All Projects" or specific data for a selected project, updating in real time as treatments are applied.

The page number shown at the top right (e.g. 3 / 120) tells you which segment you are currently viewing out of the total. Use the Previous and Next buttons, or type a page number directly, to jump to any segment.

### Step 3: Choose By Segment or By Treatment view

The Treatment Options panel on the left has a dropdown that lets you switch between two views:

| View | What it shows | Best used when |
| --- | --- | --- |
| By Segment | Only the treatments that are relevant to the segment you are currently viewing, sorted by how much each treatment would reduce the risk score (most improvement first) | You want to work through the path segment-by-segment and decide on the best fix for each one |
| By Treatment | All treatments that are applicable to at least one segment across the entire loaded project, sorted by how many segments each treatment improves | You want to roll out one improvement (e.g. adding signage) across as many segments as possible in one action |

### Step 4: Select and preview treatments

Tick the checkbox next to one or more treatments to select them. Selected treatments are highlighted in the panel.

[!IMPORTANT]

The **Show Post-Treatment** toggle (located in the Attributes panel header on the right) must be switched **on** to see the live preview of the post-treatment risk scores and attribute changes. If this is toggled off, only the original (pre-treatment) scores will be displayed.

Once the Show Post-Treatment toggle is enabled, selecting a treatment will show a live preview in the Scores card on the right of what the risk scores would look like if you applied those treatments to the current segment. The preview updates automatically whenever you change your selection — you do not need to click anything to trigger it.

With the toggle on, the attributes panel also updates to display exactly which attribute values would change if the selected treatments were applied, so you can understand the reasoning behind the score change.

**Note:** Selecting treatments only previews the effect — nothing is saved until you click Apply.

### Step 5: Apply treatments

When you are satisfied with your selection, click Apply (N) at the bottom of the Treatment Options panel (where *N* is the number of treatments selected):

In By Segment mode, Apply saves the selected treatments for the current segment only. The treatment colours on the After Treatment map will update for that segment immediately.

In By Treatment mode, Apply triggers a confirmation dialog listing each selected treatment and asking whether to apply it to all eligible segments across the loaded project. Confirm to apply in bulk, or cancel to go back.

If you change your mind about an applied treatment on the current segment, click the Reset button that appears for that treatment to remove it and restore the original scores for that segment.

### Step 6: Read the segment image, scores, and attributes

The middle column shows the street-level photograph for the current segment. Use Previous and Next to move through segments, or jump to a specific page number.

The right column shows two panels:

Segment Scores — displays the four CycleRAP risk scores for the current segment: Vehicle-Bicycle (VB), Bicycle-Pedestrian (BP), Single-Bicycle (SB), and Bicycle-Bicycle (BB). When treatments are selected, this panel shows the previewed scores alongside the original scores so you can compare.

Attributes panel — lists all coded attribute values for the current segment. Toggle Show Post-Treatment to switch between the original coded values and the values that would result from the selected treatments.

### Step 7: Before and After maps

The two maps at the top of the page give you a project-wide view of how treatments are changing risk across all segments:

Before Treatment map — always shows the original risk-band colouring for every segment (green = Low, yellow = Medium, orange = High, purple = Extreme). This map does not change as you apply treatments.

After Treatment map — updates in real time as you apply treatments. Segments that have been treated will shift colour to reflect their new risk band. Segments with no applied treatments remain the same colour as the Before map.

Click any segment on either map to jump directly to that segment's page in the panel below.

### Step 8: Generate AI visualisations of proposed improvements

The Treatment Options panel includes two clipboard buttons that help you create visual representations of proposed improvements using external AI image tools:

Copy prompt — copies a ready-to-use text prompt describing treatments in plain language. This button has a dropdown with two options:

Copy Applied — copies a prompt based on the treatments already applied and saved for this segment.

Copy Selected — copies a prompt based on the treatments currently ticked/selected in the panel (even if not yet applied).

Paste the prompt into an AI image generation tool (such as ChatGPT or DALL·E) to generate a visual mock-up of what the improved path might look like.

Copy image — copies the current segment photograph directly to your clipboard. Paste it into the same AI tool alongside the prompt so the tool has the actual scene to work from.

Use Copy Selected when you are exploring options and want to preview the AI image before committing. Use Copy Applied when you have already applied treatments and want to generate the final before-and-after visualisation.

### Step 9: Review the Overall Treatment Analysis

Scroll to the bottom of the page to see the Overall Treatment Analysis section. It shows two rows of five pie charts each — one row for Before Treatment and one for After Treatment:

Each pie chart covers one crash type: Overall, Vehicle-Bicycle (VB), Bicycle-Pedestrian (BP), Single-Bicycle (SB), and Bicycle-Bicycle (BB).

Each chart shows how the segments in the loaded project are distributed across the four risk bands (Low, Medium, High, Extreme).

The After Treatment row updates as you apply treatments, so you can see the project-wide impact of all your decisions at a glance.

Use these charts to get a quick sense of whether the treatments you have applied make a meaningful difference at the project level, not just for individual segments.

### Tips and common workflows

Pre and Post tagging — if you are working with a before-and-after survey pair, tag one project Pre and the other Post, then load both together. You can then compare the actual post-treatment survey against PSAT's predicted improvement.

Applying a single improvement across a whole project — switch to By Treatment view, find the treatment you want, tick it, then click Apply. The confirmation dialog will list all eligible segments; confirm to apply in bulk.

Undoing all changes — if you want to start fresh, navigate to each segment that has been treated and click Reset to remove treatments one segment at a time.

Checking what was already applied — treatments that have already been saved for the current segment are shown with a green background in the Treatment Options panel, so you can see at a glance what is already in place.

### 5.10 AI Score Preview — Before & After Treatment

When you select treatments in the Treatment Options panel, PSAT calculates a predicted score for the current segment in real time. This before/after comparison lets you evaluate the safety impact of a treatment before committing to it.

#### How the Before / After Comparison Works

Before score — the original crash type scores (BB, BP, SB, VB, Risk Score) computed from the coded attributes.

Treatment selected — ticking a treatment checkbox immediately previews the effect on the current segment's scores.

After score — PSAT re-runs the CycleRAP scoring formula with the updated attribute values to produce a predicted post-treatment score.

The difference (reduction) is shown as a ↓ value beneath each crash type score card. A green indicator confirms improvement; no arrow means the treatment did not affect that crash type.

#### Reading the Score Cards

When treatments are selected:

Each card shows the post-treatment score in large text.

A ↓ X.XX delta beneath it shows how much the score decreased.

The card background colour reflects the post-treatment band — if a treatment moved a segment from High to Medium, the card will show the Medium (yellow/orange) colour.

#### Previewing Before You Apply

[!IMPORTANT]

The **Show Post-Treatment** toggle must be switched **on** in the Attributes panel header to see the live score preview and attribute modifications. If it is toggled off, the score cards and attribute fields will continue showing the original pre-treatment values.

With the toggle enabled, tick any treatment checkbox to see the live score preview for the current segment. The scores and attributes update automatically as you change your selection — you do not need to click Apply.

**Note:** Selecting treatments only previews the effect — nothing is saved until you click **Apply**.

#### AI-Assisted Visualisation (Prompt & Image Copy)

Two clipboard buttons help you create visual representations of proposed improvements:

Copy prompt — copies a ready-to-use text prompt describing the selected or applied treatments. Use the dropdown to choose:

Copy Applied — prompt based on treatments already applied and saved for this segment.

Copy Selected — prompt based on treatments currently ticked/selected in the panel.

Paste the prompt into an AI image generation tool (such as ChatGPT or DALL·E) to generate a mock-up of what the improved path might look like.

Copy image — copies the current segment photograph directly to your clipboard. Paste it alongside the prompt so the AI tool has the actual scene to work from.

Use Copy Selected when exploring options before committing. Use Copy Applied when generating the final before-and-after visualisation.

# 6. Report Generation

## Overview

The Report Builder compiles a formatted, printable report of your path safety assessment. It pulls live data from the projects you have loaded in the Path Analysis page, so your active filters and selected segments are reflected automatically.

Access it from the sidebar while on the Path Analysis page, or from the Treatment page — click "Generate Report" or "Continue Report" (if you have a saved layout).

Your report layout, section titles, and editable text fields are **auto-saved to your browser**. If you close the tab and come back, click **Restore Saved** to pick up where you left off.

## How the Report is Structured

The report is built as a vertical stack of sections (blocks). Each section renders on an A4-proportioned canvas. You can:

Drag sections up or down to reorder them

Show / hide any section using the toggle on its right edge

Rename any section title by clicking on the title text inside the section

Export as PDF or Export as Word (.docx) using the top toolbar buttons

The default page order is:

| Page | Sections included |
| --- | --- |
| 1 | Title, Summary, Map |
| 2 | Risk Bands, Benchmarking Stats, Top Risk Stretches |
| 3 | Treatments |
| Off by default | Project Details, Risk Factors |

## Section 1 — Project Details (Title Page)

What it shows: Project name(s), image survey date, report date, officer-in-charge, and purpose of the report.

Where it appears: The Title section at the top of page 1.

### How to fill it in

In the top toolbar, locate the editable fields:

Report Title — click to type your own title (e.g. "Toa Payoh Path Safety Report 3Q25").

OIC Name — officer or team responsible for the report.

Purpose — brief statement of why the assessment was conducted.

Report Date — defaults to today; click to change.

Image Date — the date the survey images were taken; click to type (e.g. "Sep 2025").

Click outside a field or press Enter to confirm each change.

In the Title section itself, click on the large title text to rename it inline.

How it looks in the report: A header block showing the report title in large text, with the project name(s), survey date, report date, officer name and purpose listed below it.

**Project names** shown in the report come from whichever projects you loaded into the Path Analysis page. To load projects, return to the Path Analysis page and select projects, then come back to the Report Builder.

## Section 2 — Overall Project Risk Scores (Risk Bands)

What it shows: The distribution of segments across risk bands (Low / Medium / High / Extreme) for all five risk types: Vehicle–Bicycle (VB), Bicycle–Bicycle (BB), Single-Bicycle (SB), Bicycle–Pedestrian (BP), and Overall Risk.

### How to use it

The Risk Bands section renders automatically once your projects are loaded. No manual steps required.

Each crash type is shown as a horizontal bar chart: one bar per band, with segment count and percentage.

The colour coding is:

🟢 Low — green

🟡 Medium — yellow

🟠 High — orange

🟣 Extreme — purple

How it looks: Five stacked bar charts (VB, BB, SB, BP, Overall), one per crash type. Each bar shows how many segments fall into each risk band out of the total segment count.

**Tip:** The Risk Bands section reflects your **active filters**. If you have filtered the Path Analysis page to only show certain segments (e.g. by road type or rating), only those segments appear in the bands.

## Section 3 — Project Analysis Report (Map + Summary + Top Segments)

This is split across three sub-sections that together form the full analysis report.

### 3a — Summary

What it shows: Total segment count, active filter summary, and the project(s) included.

Loads automatically when projects are selected.

If you have filters active on the Path Analysis page, the filter names appear here to record exactly what was analysed.

### 3b — Map

What it shows: A live route map with every segment plotted as a colour-coded dot, matching the risk band colours above.

The map renders automatically from your project geodata.

Segments are coloured by their Overall Risk Level Band.

The map zooms to fit all loaded segments.

The map is interactive in preview but is captured as a static image in the PDF/Word export.

### 3c — Top Risk Stretches

What it shows: The highest-scoring segments ranked from worst to best, with images, scores, contributing factors, and any treatments applied.

#### How to configure

In the Top Risk Stretches section, use the View toggle at the bottom:

Full Page — one segment per page, large image + full detail (recommended for reports).

Grid — compact card grid (3 per row).

Tabular — compact table with thumbnails.

Use the Top N selector to choose how many segments to show: 3 to 10 (default is 10).

#### What each segment card shows (Full Page view)

Each of the top 10 segments gets its own full page with:

| Element | Description |
| --- | --- |
| Ranking badge | Large numbered circle (1 = worst) |
| Survey image | Actual photo of that location |
| Project name | Which project the segment belongs to |
| Risk Score | Combined score (large number, colour = band) |
| Top Contributing Attribute | The single attribute contributing most to the risk score, with its multiplier value |
| Other significant factors | Up to 4 additional contributing attributes |
| Applied Treatments | Green box listing any treatments already applied to this segment |
| Crash type breakdown | VB / BB / SB / BP scores and their individual band colours |

Contributing factors are identified from the **CycleRAP scoring multipliers** — each attribute that raises the risk score is listed with how much it contributed (shown as a negative number, e.g. −3.5).

## Section 4 — Project Treatment Report (Before & After)

What it shows: Which treatments were applied across the project(s) and a summary of how many segments were treated per treatment type.

### How the before/after works

Treatments must first be applied in the Treatment Page (accessed from the sidebar on the path analysis or projects page).

After applying treatments and returning to the Report Builder, the Treatments section lists:

Each treatment type (e.g. "Install street lighting", "Widen the facility").

How many segments received each treatment.

The Top Risk Stretches section (above) shows the Applied Treatments box on each segment card — this is the "after" side of the before/after comparison. The segment image shown is the before survey photo.

### Pasting AI-generated "after" images

For each of the top 10 segments, the report shows the real survey photo as the before image. To add an AI-generated after image showing what the location could look like post-treatment:

In the Treatment Page, use the AI Prompt feature to generate an after-treatment visualisation for that segment.

Copy the AI-generated image to your clipboard (right-click → Copy Image, or use the copy button if available).

In the Report Builder, open the top risk segment you want to update.

Paste the image directly into the segment image area — click the image area once to select it, then press Ctrl+V.

The pasted AI image replaces the survey photo for that segment in the report export.

Currently the paste-in feature works in **Full Page** view mode. Ensure you are in Full Page mode before pasting.

## Section 5 — Benchmarking (Verified Projects)

What it shows: How the current project(s) compare against all other verified projects in the system — showing min, average, and max scores for each crash type across the verified set.

### How to read it

| Column | Meaning |
| --- | --- |
| Min | Lowest score seen across all verified projects |
| Avg | Average score across verified projects |
| Max | Highest score seen across verified projects |
| Your project | The current project's average score (highlighted) |

The five rows are: VB, BB, SB, BP, and Overall Risk.

### How benchmarking works

A project is included in the benchmark pool only if it has been verified (the Verified toggle in the Projects page must be on).

The benchmarking section loads automatically — no extra steps needed.

Use this to see whether your project's risk scores are above or below typical values for similar paths in the dataset.

If no other verified projects exist in the system, the benchmarking section will show only your current project's data.

## Saving, Exporting and Reusing Your Report

### Save your layout

Click Save Layout in the toolbar. Your section order, section titles, all editable text (report title, OIC, purpose, dates), and any customisations are saved to the browser.

### Export as PDF

Click Export PDF. The report renders exactly as shown in the preview, sliced into A4 pages. Each section begins on a clean page boundary.

### Export as Word (.docx)

Click Export Word. A .docx file is generated with the same content, suitable for further editing in Microsoft Word.

### Restore a previous layout

Click Restore Saved on the toolbar to reload a layout you saved earlier in the same browser.

### Reset to default

Click Reset to clear all customisations and return to the default section layout.

## Quick-start Steps

Load your projects in the Path Analysis page; apply any filters you need.

Open Report Builder from the sidebar ("Generate Report").

Fill in the Title toolbar fields: Report Title, OIC Name, Image Date, Report Date, Purpose.

Toggle sections on/off as needed; drag to reorder.

Set Top Risk Stretches to Full Page, Top 10.

(Optional) Paste AI-generated after images into segment image areas.

Click Export PDF or Export Word.

# 7. GIS Layer Management

PSAT uses external GIS shapefiles and GeoJSON layers to provide spatial context for path coding and risk analysis. Use the GIS Layer Management tool to keep these datasets up to date.

## Table of Contents

## 7.1 Viewing GIS Layers

Click the View GIS Layers button in the sidebar to open the GIS Layers dashboard. Here you can:

Browse all current GIS layers (sorted in alphabetical order)

View last updated date, required columns, and metadata for each layer

Required Columns: Inspect the mandatory column names needed for PSAT to process the layer. The number in parentheses, e.g., LU_DESC (1), indicates the expected column index in the source data

Preview any layer on an interactive map by clicking it

Filter the list instantly by typing a keyword into the filter box (see Section 7.6)

Rename any layer to a custom display name or revert it back to its original name (see Sections 7.7 and 7.8)

## 7.2 All GIS Layers in PSAT

The table below lists every shapefile currently loaded in PSAT, the PSAT attribute it affects, and the required columns. Layer names match the display names shown on the GIS Layers page.

| Layer Name | Category | Geometry | PSAT Attribute Affected | Required Columns |
| --- | --- | --- | --- | --- |
| 31Oct24 Link Ful | LinkID_Shape_File | LineString | Road Operating Speed (mean) | LK_ID_NUM (1) |
| 31Oct24 Node Ful | LinkID_Shape_File | Point | Road Network Node Reference | NODE_ID_NU (1), RD_TYP_CD (2) |
| Amgbeforecount Export | AMGbeforeCount | Point | Peak Pedestrian Flow, Peak Bicycle Traffic Flow | DataType (1), DateTime (2), Count_Data (3) |
| Amgsensorcount Export | AMGsensorCount | Point | Peak Pedestrian Flow, Peak Bicycle Traffic Flow | Pivot_user (1), Datetime_p (2), Count (3) |
| Bus Lanes | bus_lane | LineString | Heavy Vehicle Flow; Extracted Type: A1, A6, A7, L, Q, Q1, Q2, Y | TYP_CD (1), TYP_NAM (2) |
| Busshelter | bus_stop | LineString | Pedestrian Crossing, Peak Flow | BUS_ROOF_N (1), LOC_DESC (2) |
| Busstop | bus_stop | Point | Pedestrian Crossing, Peak Flow | BUS_STOP_N (1), LOC_DESC (3) |
| Centralmb2025 | area_type | Polygon | Area Type (Urban) | PARKING_ZO (1), STATION_NA (2) |
| Cyclingpathcentreline | path | LineString | Facility Width, Curvature | WIDTH (1) |
| Cyclingpathgazette | CyclingPath_Jul2024 | LineString | Planning Area Reference | PLANNING_A (1), PLANNING_1 (2) |
| Dgp 27Oct14 | Planning_area | Polygon | Area-based Reporting | PLN_AREA_N (1) |
| Existing Cycling Paths | CyclingPath_Jul2024 | LineString | Cycling Path Type Reference | path_type (1) |
| Footpath | FootPath_Mar2025 | LineString | Facility Width, Curvature | WDT_CATG_C (1), TYP_CD (2) |
| Footpathcentreline | path | LineString | Facility Width, Curvature | WIDTH (1) |
| Hdb Car Park Lots | parking_lot | Point / Polygon | Adjacent Vehicle Parking | CARPARKNO (1), NOOFCARLOT (2) |
| Kerbline | kerb_line | LineString | Number of Lanes – Adjacent Road | LANES (1), LOCATION (2), DIRECTION (3) |
| Landuseindustrial2025 | area_type | Polygon | Area Type (Urban, Industrial, Rural, Recreational) | LU_DESC (1), LU_TEXT (3) |
| Landuserecre2026 | LanduseRecre2026 | Polygon | Area Type (Recreational) | LU_DESC (1), LU_TEXT (3) |
| Landuserural2025 | area_type | Polygon | Area Type (Urban, Industrial, Rural, Recreational) | LU_DESC (1), LU_TEXT (3) |
| Landuserural2026 | LanduseRural2026 | Polygon | Area Type (Rural) | LU_DESC (1), LU_TEXT (3) |
| Mrt Exits | Mrt_exit | Point | Pedestrian Crossing, Peak Flow | STATION_NA (1), EXIT_CODE (2) |
| Roadattributeline Speedlimits | Speed_limit | LineString | Road Speed Limit | SPEEDLIMIT (1) |
| Roadcrossing | roadcrossinglayer | LineString / Point | Pedestrian Crossing | UNIQUE_ID (1) |
| Roadnetworkline | Road_name | LineString | Road Name Reference | RD_CD_DESC (1) |
| Sharedpathcentreline | path | LineString | Facility Width, Curvature | WIDTH (1) |
| Ura Parking Lot | parking_lot | Polygon | Adjacent Vehicle Parking | PP_CODE (1), LOT_NO (2), TYPE (3) |

**Column index numbers** in parentheses indicate the column position (1-based) expected in the source shapefile. These are used during validation when uploading replacement layers.

## 7.3 Adding a New GIS Layer

Use the Add GIS Layer workflow to upload entirely new datasets.

New Category: You must provide a name for the category folder (e.g., school_zones). Every upload is assigned to a specific category.

File Upload: Drag and drop your GIS files. For shapefiles, ensure you upload all companion files together (.shp, .shx, .dbf, .prj).

Preview: Once uploaded, you can preview the geometry on the map before finalising.

### Before You Upload — Confirmation Prompt

Before the file is accepted, PSAT displays a "Before You Upload" confirmation dialog. You must confirm that your shapefile meets all of the following requirements:

| Requirement | Description |
| --- | --- |
| Exact file name | The file name must exactly match what the system expects. |
| Exact columns and numbers | The shapefile must contain the correct number of columns, with no extras or omissions. |
| Exact attribute names | Every column/attribute name must match exactly, including capitalisation. |
| Exact sequence | The columns must appear in exactly the same order as specified. |

**Warning:** Uploading an incompatible shapefile may cause system errors or incorrect data rendering.

If you are unsure of the expected format, refer to the existing shapefiles in the GIS Layers list as a reference before uploading.

Click Confirm & Upload to proceed, or Cancel to go back and check your file.

## 7.4 Replacing an Existing GIS Layer

Use the Replace GIS Layer workflow when you have updated data for an existing layer.

Filterable Search: Use the searchable inputs to quickly find the folder and specific layer you wish to replace. Type a few letters (e.g., bus) to filter the list.

Safety Checks: PSAT performs compatibility checks to ensure the new file has the same required columns as the original.

Warnings: If differences are found in the column structure, the system will warn you before overwriting the old data.

## 7.5 Updating GIS Layers

GIS layers should be updated whenever:

A new version of a dataset is released (e.g., updated cycling path network)

A new category of infrastructure needs to be tracked

The existing layer has outdated or incorrect geometry

Steps to update a layer:

Obtain the new shapefile from the relevant data source (e.g., LTA, URA)

Verify the file has the required columns listed in the table above

Open GIS Layers from the sidebar

Use Replace GIS Layer, search for the existing layer, and upload the new file

Review the compatibility check results before confirming the replacement

For system-level GIS management (file paths, permissions, bulk updates), refer to the **Admin Guide**.

## 7.6 Filtering Shapefiles

When your system has many GIS layers, scrolling through the full list can be slow. The filter box at the top of the Available Shapefiles panel lets you narrow the list instantly.

How to use it:

Open the GIS Layers page from the sidebar

Click the Filter shapefiles… input field beside the "Available Shapefiles" heading

Type any keyword — the list updates in real time to show only layers whose name contains your search term (the search is case-insensitive)

Clear the field to restore the full list

Examples:

| What you type | Layers shown |
| --- | --- |
| bus | bus_stop, bus_lane, bus_shelter … |
| path | cycling_path, shared_path, footpath, CyclingPath_Jul2024 … |
| count | AMGbeforeCount, AMGsensorCount … |

If no layers match your search term, the panel displays a "No shapefiles match …" message.

## 7.7 Renaming a Shapefile

You can give any shapefile a custom display name to make it easier to identify within your team's workflow. The rename only affects how the layer appears in the PSAT dashboard — the underlying file on disk is not moved or changed.

Steps:

Open the GIS Layers page from the sidebar

Locate the shapefile you want to rename (use the filter box if needed)

Click the Edit button on the right side of the layer row

The layer name becomes an editable text field — type your new name

Press Enter or click the save (✓) button to confirm

Press Escape or click the cancel (✕) button to discard the change

Renamed layers retain all their original metadata, required columns, geometry, and category. Only the display name is updated.

## 7.8 Reverting a Shapefile to Its Original Name

If a layer has been renamed and you want to restore its original filename as the display name, use the Revert option.

Steps:

Open the GIS Layers page from the sidebar

Locate the renamed shapefile (a Revert button will be visible next to Edit)

Click Revert

Confirm the action — the display name immediately reverts to the layer's original filename

The **Revert** button only appears on layers that have been given a custom name. Layers still showing their original name will not have this option.